- Power On/Off

| Sr No. |  | Description |  | Key |
| --- | --- | --- | --- | --- |
| 1 | Orbit Player Power On/Off |  | Press and hold the power Key for two seconds to turn on the unit. |  |

# Home Application

| Sr No. |  | Description | Key |  |
| --- | --- | --- | --- | --- |
| 1 | Select Previous option |  |  | Up Arrow |
| 2 | Select Next option |  |  | Down Arrow |
| 3 | Select focused item |  |  | Select |
| 4 | Navigate to Main List |  |  | Left Arrow |
| 5 | Navigate to Sub-Menu List |  |  | Right Arrow / Back Key |
| 6 | Navigate using Keypad numbers. Press any number from the numpad and it will open that menu item. |  |  | Num Keys. |

# General Commands

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Sleep/wakeup |  | Short tap power Key (Single Tap to Power Key) |
| 2 | Power ON/OFF |  | Long tap power Key (Press Power Key Min 2 seconds) |
| 3 | Mass storage enter/exit. |  | Num 0 + Num 5 + Num 7 |
| 4 | Processor volume up |  | Right side up Key |
| 5 | Processor volume down |  | Right side down Key |
| 6 | Battery cutoff from MCU |  | For units with serial numbers up to 95 or those using hardware version 2.0 or earlier: Home Key + Num # For units with serial numbers 96 and above or those with hardware version 2.1: Vol Down Key + Num # |
| 7 | MCU hard reset |  | For units with serial numbers up to 95 or those using hardware version 2.0 or earlier: Home Key + Num 8 Key For units with serial numbers 96 and above or those with hardware version 2.1: Vol Down Key + Num 8 Key |
| 8 | DFU mode for MCU firmware upgrade. |  | For units with serial numbers up to 95 or those using hardware version 2.0 or earlier: Home Key + Num 8 Key + Num 6 Key For units with serial numbers 96 and above or those with hardware version 2.1: Vol Down Key + Num 8 + num 6 Key |
| 9 | Android Hard reset |  | For units with serial numbers up to 95 or those using hardware version 2.0 or earlier: Home Key + Star Simultaneous for 15 sec For units with serial numbers 96 and above or those with hardware version 2.1: Vol Down Key + Star Simultaneous for 15 sec |

# Battery Logging

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Navigate Up/Down in the config dialog. |  | UP Arrow / Down Arrow |
| 2 | Open config dialog. |  | Menu + Record + Star + Hash + Select |
| 3 | Start/Stop logging. |  | Menu + Record + Star + Hash |

# Global Hot Keys

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Check Battery Status |  | Menu + Record |
| 2 | Keyboard help feature |  | Menu + Num 1 |
| 3 | Decrease speech volume |  | Menu + Num 2 |
| 4 | Increase speech volume. |  | Menu + Num 3 |
| 5 | Opens speech settings. |  | Menu + Num 4 |
| 6 | Decrease speech rate. |  | Menu + Num 5 |
| 7 | Increase speech rate. |  | Menu + Num 6 |
| 8 | Decrease speech pitch. |  | Menu + Num 7 |
| 9 | Increase speech pitch. |  | Menu + Num 8 |
| 10 | Speak current time, if 9 pressed twice speaks current date. |  | Menu + Num 9 |
| 11 | Device lock/unlock. |  | Menu + Num * |
| 12 | Opens the settings application. |  | Menu + Num 0 |
| 13 | Speaks wi-fi & Bluetooth status. |  | Menu + Num # |
| 14 | Change the mode (Letter Navigation / Command Mode) |  | Long Num * |
| 15 | Toggle options (Speech volume, device volume, speech rate, pitch rate) to control using volume Keys |  | Volume Up + Volume Down |
| 16 | Perform increase/decrease function according to the selected option. |  | Volume Up / Volume Down |

File Manager Commands

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | File Manager |  | Select/ Press 4 (from utilities sub-menu) |
| 2 | Exit File Manager |  | Home Key / Back Key |
| 3 | Select Prev/Next File |  | Up/Down Arrow |
| 4 | Open File/Folder |  | Select |
| 5 | File/Folder Properties |  | Num * |
| 6 | Navigate to previous folder |  | Left Arrow |
| 7 | Open Context Menu |  | Menu Key |
| 8 | Close Context Menu or close any other dialog |  | Back Key |
| 9 | Mark/Unmark File/Folder |  | Num 0 |
| 10 | Confirm Delete |  | Select |
| 11 | Open delete file/folder Dialog |  | Long Press Back Key |
| 12 | Navigate Up/Down in any menu (Context, Delete dialog, etc.) |  | Up/Down Arrow Key |
| 13 | Top / Bottom of list |  | Up Long / Down Long Key |

# Settings

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Open sub-settings |  | Right arrow / Select Key |
| 2 | Navigate to different combo box |  | Up/Down arrows |
| 3 | Navigate between the values in the combo box |  | Left/Right arrow |
| 4 | Apply all combo box values |  | Select |
| 5 | Close dialog |  | Back Key |
| 6 | Top / Bottom in the list. |  | Up Long/ Down Long Key |

# Wi-Fi Setting

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Navigate through available network list |  | Up/Down Arrows |
| 2 | Perform action |  | Select Key (by focusing that item) |
| 3 | Enable/Disable Wi-Fi |  | Select Key (by focusing that item) |
| 4 | Refresh available network list |  | Select Key (by focusing that item) |
| 5 | Context Menu: Open |  | Select Key (by focusing the network) / Menu Key |
| 6 | Context Menu: Close |  | Back Key |
| 7 | Close Wi-Fi Settings dialog |  | Back Key |
| 8 | Top / Bottom in the list. |  | Up Long/ Down Long Key |

# Bluetooth Setting

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Navigate through available device list |  | Up/Down Arrows. |
| 2 | Perform action |  | Select Key. |
| 3 | Enable/Disable Bluetooth |  | Select Key (by focusing that item) |
| 4 | Refresh available device list |  | Select Key (by focusing that item) |
| 5 | Context Menu: Open |  | Select Key (by focusing the network) / Menu Key |
| 6 | Context Menu: Close |  | Back Key. |
| 7 | Close Bluetooth Settings dialog |  | Back Key. |
| 8 | Top / Bottom in the list. |  | Up Long/ Down Long Key |

# Calculator

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Remove Character |  | Back Key (when focused on edit field) |
| 2 | Evaluate expression (Get Result) |  | Select Key |
| 3 | Open Context Menu |  | Menu Key |
| 4 | Close Context Menu |  | Back Key |
| 5 | Navigate up/down in context menu |  | Up/Down Arrow |
| 6 | Exit from app |  | Back Key / Long Press to back Key when focused item is editing field. |
| 7 | Top / Bottom in the menu items. |  | Up Long/ Down Long Key |

# Alarm Clock

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Navigate Up/Down in alarm list, edit alarm dialog options. |  | Up Arrow / Down Arrow |
| 2 | Navigate Up/Down in the new alarm dialog options or delete alarm dialog option. |  | Up Arrow / Down Arrow |
| 3 | Open Context Menu |  | Menu Key |
| 4 | Navigate up/down in context menu |  | Up Arrow / Down Arrow |
| 5 | Exit from any dialog |  | Back Key |
| 6 | Exit from app |  | Back Key |
| 7 | Toggle Alarm |  | Num 0 Key |
| 8 | Navigate items in the combo box in the new alarm dialog & edit alarm dialog. |  | Left Arrow / Right Arrow |
| 9 | Navigate Up/Down in the Alarm triggering dialog. |  | Up Arrow / Down Arrow |
| 10 | Select option in alarm triggering dialog or delete dialog |  | Select Key |
| 11 | Exit App |  | Back Key |
| 12 | Dismiss all alarms |  | Long press on Back Key |
| 13 | Dismiss the triggering alarm |  | Back Key |
| 14 | Top / Bottom of the list, menu items, new alarm, edit alarm dialog. |  | Up Long/ Down Long Key |

# Contacts

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Navigate Up/Down in contact list. |  | Up Arrow / Down Arrow |
| 2 | Navigate Up/Down in dialog |  | Up Arrow / Down Arrow |
| 3 | Open Context Menu |  | Menu Key |
| 4 | Navigate up/down in context menu |  | Up Arrow / Down Arrow |
| 5 | Exit from any dialog |  | Back Key |
| 6 | Exit from app |  | Back Key |
| 7 | View Contact |  | Select Key |
| 8 | Save changes in edit and new contact dialog |  | Select Key |
| 9 | Exit when focused on the edit field |  | Long press of back Key. |
| 10 | Top / Bottom in the contact list, menu items. |  | Up Long/ Down Long Key |

# Calendar

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Navigate Up/Down/Left/Right in calendar view. |  | Up Arrow / Down Arrow / Left Arrow / Right Arrow |
| 2 | Open Context Menu |  | Menu Key |
| 3 | Access Key of Exit App in context menu |  | Back Key |
| 4 | Navigate up/down in the new event, edit event, context menu, go to date, delete event, event triggered dialog & event details page. |  | Up Arrow / Down Arrow |
| 5 | Exit from any dialog |  | Back Key |
| 6 | Exit from app |  | Back Key |
| 7 | Navigate items in the combo box in the new event dialog, edit event dialog or go to date. |  | Left Arrow / Right Arrow |
| 8 | Navigate week wise in the calendar view. |  | UP Arrow / Down Arrow |
| 9 | Navigate to previous/next date in the calendar view. |  | Left Arrow / Right Arrow |
| 10 | Select option in event triggering dialog, delete dialog, new event dialog, edit event dialog, go to date, context menu dialog. |  | Select Key. |
| 11 | Dismiss the triggering alarm |  | Back Key. |
| 12 | Open Event Details page. |  | Select Key |
| 13 | Navigate back from the edit field focused. |  | Long back Key. |
| 14 | Navigate top / bottom in the new event, edit event, context menu, go to date, delete event, event triggered dialog & event details page. |  | Long Up Key / Long Down Key |

# Internet Radio

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Navigate between category list and item list |  | Left/Right Arrow. |
| 2 | Navigate between any list like category list, country list or station list. |  | Up/Down Arrow. |
| 3 | Close Internet Radio |  | Back Key. |
| 4 | Navigate in current list |  | Up/Down Arrow |
| 5 | Select item in current list |  | Select Key |
| 6 | Open Context Menu |  | Menu Key |
| 7 | Navigate back when focusing in on edit field. |  | Long press of back Key. |
| 8 | Close context menu |  | Back Key |
| 9 | Player Dialog: Play/Stop |  | Select Key |
| 10 | Close dialogs |  | Back Key |
| 11 | Navigate from item list to category list. |  | Back Key |
| 12 | Start/Stop recording (only when the station is playing) |  | Record Key |
| 13 | Increase/Decrease volume |  | Num 6 or Volume Up / Num 4 or Volume Down |
| 14 | Navigate to Top/Bottom of the list or menu. |  | Long Up Key / Long Down Key |

# Reader

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Navigate between category list and item list |  | Left/Right Arrow. |
| 2 | Close Reader |  | Back Key. |
| 3 | Navigate in current list |  | Up/Down Arrow Key. |
| 4 | Open book or Select item in current list |  | Select Key. |
| 5 | Open context menu |  | Menu Key |
| 6 | Close context menu |  | Back Key. |
| 7 | Book Reader Dialog: Play/Stop |  | Select Key. |
| 8 | Close dialogs |  | Back Key. |
| 9 | Navigate to Top/Bottom of the list. |  | Long Up Key / Long Down Key |
| 10 | Increase/Decrease volume |  | Num 6 or Volume Up / Num 4 or Volume Down |
| 11 | Change playback state in the reader dialog. |  | Select Key. |
| 12 | Decrease/Increase navigation mode in the reader dialog. |  | Left / Right Key. |
| 13 | Increase/Decrease playback speed in the reader dialog. |  | Num 9 / Num 7 |
| 14 | Start of file/End of file. |  | Up long Key / Down long Key |
| 15 | Previous / Next option in the combo box in any dialog such as go to location dialog, settings dialog, speech settings dialog. |  | Left Key / Right Key. |
| 16 | Perform any action. |  | Select Key. |

# Media Player Application Commands

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Open File Manager |  | Long press Num 0 |
| 2 | Play/Pause |  | Select |
| 3 | Perform previous action according to the movement unit |  | Right |
| 4 | Perform next action according to the movement unit |  | Left |
| 5 | Previous Track |  | Num 1 |
| 6 | Next Track |  | Num 3 |
| 7 | Shuffle Mode toggle |  | Num 2 |
| 8 | Track Name Info |  | Hash Key |
| 9 | Open Context Menu |  | Menu Key |
| 10 | Navigate Context Menu Items |  | Up Arrow / Down Arrow |
| 11 | Navigate to previous movement unit |  | Up Arrow |
| 12 | Navigate to next movement unit |  | Down Arrow |
| 13 | Increase/Decrease volume |  | Num 6 or Volume Up / Num 4 or Volume Down |
| 14 | Increase/Decrease playback speed |  | Num 9 / Num 7 |
| 15 | Close Context Menu or Settings |  | Back Key |
| 16 | Close Media Player app |  | Back Key |
| 17 | Time info |  | Num 0 |
| 18 | Settings dialog |  | Num * |
| 19 | Change combo box options in the settings dialog |  | Left/Right Key |
| 20 | Toggle Repeat Mode |  | Num 5 |
| 21 | Top / Bottom in the context menu or settings. |  | Up Long Key / Down Long Key |

# Voice Recorder Commands

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Open Recorder |  | Select Key |
| 2 | Navigate in the menu, player dialog, settings dialog, recorder view, recordings. |  | Up/Down Arrow Key |
| 3 | Select any item |  | Select Key |
| 4 | Open context menu |  | Menu Key |
| 5 | Close context menu or player dialog |  | Back Key |
| 6 | Navigate to the items in the combo box of the settings dialog. |  | Left/Right Key |
| 7 | Close app |  | Back Key. |
| 8 | Close from the dialog when focus is on edit field |  | Long press of back Key. |
| 9 | Start/Stop recording |  | Recorder Key. |
| 10 | Top / Bottom in the context menu, recording list. |  | Up Long Key / Down Long Key |
| 11 | Increase/Decrease volume |  | Num 6 or Volume Up Key / Num 4 or Volume Down Key |
| 12 | Increase/Decrease speed in the player |  | Num 9 / Num 7 |
| 13 | Announce Time |  | Num 0 |
| 14 | Announce dialog (Currently playing) |  | Key # |
| 15 | Toggle state in the player |  | Select Key |

# Software Updater Commands

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Navigate Up / Down |  | Up Key / Down Key |
| 2 | Select any item to perform an action |  | Select Key |
| 3 | Close app |  | Back Key |

# Speech Options Commands

| Sr No. |  | Description | Key |
| --- | --- | --- | --- |
| 1 | Navigate Up / Down |  | Up Key / Down Key |
| 2 | Select any item to perform an action |  | Select Key |
| 3 | Close app |  | Back Key |
| 4 | Navigate to the items in the combo box. |  | Left / Right Key |

©2025 Orbit Research. All rights reserved.